Every OTA update must include native firmware built with NF_PRODUCT_OTA_ALLOW_VS_DEPLOY=ON. The steps below keep Visual Studio deployment enabled, retain signed OTA updates, and leave Secure Boot disabled.

The resulting update.bin contains both the firmware and your managed application.

1. Build the application — Windows PowerShell

$ErrorActionPreference = "Stop"

$nfRoot = "C:\Users\MMDev\Documents\GitHub\nf-interpreter"

Set-Location "C:\Users\MMDev\Documents\GitHub\SeadragonS3"

.\firmware\ota\build-managed.ps1 -NFInterpreterRoot $nfRoot

New-Item -ItemType Directory -Force `

"$nfRoot\build\ota-validation" | Out-Null

Copy-Item ".\SeadragonS3\bin\Release\application.bin" `

"$nfRoot\build\ota-validation\application.bin" -Force

2. Build the matching firmware — ESP32 dev-container terminal

Use a new release ID for every OTA attempt, including retries after a failed update. IDs must contain 1–16 characters without spaces. The example below uses 20261021-02; use the same ID in step 3.

cd /workspaces/nf-interpreter

python -m pip install -r tools/product_ota/requirements.txt

cmake --preset PRODUCT_S3_OTA \

-B build/product_s3_ota_update \

-DCMAKE_BUILD_TYPE=MinSizeRel \

-DNF_USER_KCONFIG_OVERLAY="" \

-DNF_PRODUCT_OTA_ALLOW_VS_DEPLOY=ON \

-DNF_PRODUCT_MANAGED_IMAGE="$PWD/build/ota-validation/application.bin" \

-DNF_PRODUCT_RELEASE_ID=20261021-02 \

-DNF_PRODUCT_PUBLIC_KEY="$PWD/config/product-ota-public.pem" \

-DNF_PRODUCT_CONFIG_DIRECTORY=""

cmake --build build/product_s3_ota_update --parallel 8

Continue only when the build succeeds. This product preset enables the USB debugger transport and requires signed OTA packages.

3. Create and verify the signed OTA file — Windows PowerShell

Use the existing signing key matching the public key installed in the device:

$ErrorActionPreference = "Stop"

Set-Location "C:\Users\MMDev\Documents\GitHub\nf-interpreter"

py -m pip install -r tools/product_ota/requirements.txt

if ($LASTEXITCODE -ne 0) { throw "Dependency installation failed" }

py tools/product_ota/product_ota.py package `

--runtime build/product_s3_ota_update/nanoCLR.bin `

--managed build/ota-validation/application.bin `

--release-id 20261021-02 `

--private-key "C:\Users\MMDev\.seadragon-ota\keys\production-2026-10-21\seadragon-ota-private.pem" `

--output build/product_s3_ota_update/update.bin

if ($LASTEXITCODE -ne 0) { throw "OTA packaging failed" }

$manifest = Get-Content `

"build/product_s3_ota_update/update.bin.manifest.json" `

-Raw | ConvertFrom-Json

py tools/product_ota/product_ota.py verify `

--package build/product_s3_ota_update/update.bin `

--public-key config/product-ota-public.pem `

--expected-crc $manifest.package.crc32

if ($LASTEXITCODE -ne 0) { throw "OTA verification failed" }

$manifest.package | Format-List length, crc32

4. Send this file through your existing OTA protocol

C:\Users\MMDev\Documents\GitHub\nf-interpreter\build\product_s3_ota_update\update.bin

Use model Seadragon-S3 and the complete package’s crc32 from the manifest. Routine OTA requires no mass erase; it preserves the existing configuration filesystem.

After the update, let the application finish startup and confirm the new firmware before deploying or debugging from Visual Studio.

For subsequent updates, repeat all three build/package steps—even for application-only changes. Keep the VS deployment flag ON, and keep the release ID and managed binary identical between the native build and packaging steps.